MECH 337
Thermodynamics 
week 7
 
design of the week




d e s i g n    o f   t h e    w e e k
[image: image1.png]



Organic Rankine Cycle

Cummins Waste Heat Recovery Project for HDDV
The goal of this Department of Energy project is to increase the efficiency of on-road heavy duty diesel vehicles (HDDV’s) by > 10%.   One method of achieving these goals is to recovery energy from the exhaust gases and/or engine block and use that energy to power a Rankine Cycle.   In this type of application, TH is much lower than that available in a typical steam power plant and therefore steam is not used as the working fluid.  Instead a refrigerant is used as the working fluid (hence the name “organic” Rankine Cycle, since these refrigerant molecules typically contain carbon).

1.  Using R-134a as a working fluid, estimate the maximum amount of power (kW) that you can extract from the engine block by replacing the engine liquid coolant loop of a 300 HP engine with an Organic Rankine Cycle.  Calculate the thermal efficiency of such a Rankine Cycle.  Make some quick and (to your best ability) reasonable assumptions.
2. Given this maximum theoretical amount of additional power, is this concept economically viable? 

Diagram A – Current System

                 Diagram B – Proposed System
[image: image2.wmf]Engine Block

Engine Block

Radiator

Pump

Engine Block

Q

in

.

Q

out

.

water

         [image: image3.wmf]Engine Block

Engine Block

Turbine

Condenser

Pump

Engine Block

Boiler

Q

in

.

Q

out

.

W

t

.

1

2

3

4

R134a

W

p

.











