FE3.9 Water flowing in a smooth 6-cm-diameter pipe enters a venturi contraction with a throat diameter of 3 cm. Upstream pressure is 120 kPa. If cavitation occurs in the throat at a flow rate of 155 gal/min, what is the estimated fluid vapor pressure, assuming ideal frictionless flow?

(a) 6 kPa

(b) 12 kPa

(c) 24 kPa

(d) 31 kPa

(e) 52 kPa

FE3.10        Water flowing in a smooth 6-cm-diameter pipe enters a ventun contraction with a throat diameter of 4 cm. Upstream pressure is 120 kPa. If the pressure in the throat is 50 kPa, what is the flow rate, assuming ideal frictionless flow?

(a) 7.5 gal/min (b) 236 gal/min     (e) 263 2  gal/min       (d) 745 gal/min (e) 1053 gal/min